LINDLEY	SYSTEMS

4257 Berwick Place

Woodbridge, VA 22192

(703) 590-8890

INTERFACE CONNECTION &

DRIVER REASSEMBLY GUIDE

-----------------------------------------------------------------------------

Configure all printers (via DIP switches usually) as follows:

No Linefeed on Carriage Return

No Parity Check (Serial interfaces)

-----------------------------------------------------------------------------

8250 Serial Interface (H/Z-89/90 and H-8-4 board)

** METHOD 1 **

Most printers will use this configuration. [Early Epson printers use

this if JNOR is connected.]  Use a one-to-one standard RS-232 cable

with a male connector on both ends or make the following connections:

Printer H89			H8-4 Molex

3	 3   Received Data	 5

7	 7   Signal Ground	13

20	20   Data Set Ready	14

With this wiring, use the default MASK = 040 and READY = 040.

Note: This wiring is most convenient as	a special cable	is not

required.

** METHOD 2 **

Some printers use this configuration.

[Early Epsons if JNOR is disconnected and JREV is connected.]

Printer H89			H8-4 Molex

3	 3   Received Data	 5

7	 7   Signal Ground	13

20*	 4   Clear To Send	 7

*For Okidata Microline and MPI printers use pin 11 instead of 20 here.

With this wiring, use SET to set the MASK = 020	and READY = 000.

Note: This wiring is compatible	with some Heath/Zenith drivers,

such as	LPH24.DVD and CP/M BIOS, but needs a special cable.

You can	also jumper pins 4 and 11 of the RS232 connecter in the

printer	and wire pins 3, 4, 7, and 20 between the computer and

printer	to be compatible with Magnolia CP/M.

H-8-2 parallel or H-8-5	serial board with 8251 UART:

Driver Reassembly:

Change the value of U8251 to TRUE, and all others to FALSE.

(File LPSWITCH.ACM)

Printer	to H-8-2 Cable Connections:

PARALLEL		  P-101

PRINTER	  PIN		  PIN	       H8-2

-----------------		  -----------------

/STROBE. . . .	1 ----------- 11  TAKE DATA

* 19 -----------  9  *

DATA 1 . . . .	2 -----------  1  DATA 1

* 20 -----------  9  *

DATA 2 . . . .	3 -----------  2  DATA 2

* 21 -----------  9  *

DATA 3 . . . .	4 -----------  3  DATA 3

* 22 -----------  9  *

DATA 4 . . . .	5 -----------  4  DATA 4

* 23 -----------  9  *

DATA 5 . . . .	6 -----------  5  DATA 5

* 24 -----------  9  *

DATA 6 . . . .	7 -----------  6  DATA 6

* 25 -----------  9  *

DATA 7 . . . .	8 -----------  7  DATA 7

* 26 -----------  9  *

DATA 8 . . . .	9 -----------  8  DATA 8

* 27 -----------  9  *

BUSY . . . . . 11 ----------- 10  DATA TAKEN

* 29 -----------  9  *

* Indicates ground wires if ribbon cable is used.

Otherwise, use appripriate signal return.

Data lines are high true.

/STROBE is true low. On high to low transition, printer accepts data.

DATA TAKEN completes the parallel output action	and sets the TXRDY

bit to show that the 8251 can take another byte.

The 8251 Transmitter ready bit is used to test for ready.

For the	H8-2 parallel interface, set the the jumpers as	follows:

A1 - A2	OPEN		All interrupts open.

B1 - B2	CONNECTED

C1 - C2	CONNECTED

C1 - C3	OPEN

E1 - E2	CONNECTED

E1 - E3	OPEN

F1 - F2	OPEN

G1 - G2	OPEN

H1 - H2	OPEN

With the H8-5 board, no	handshake is provided.	Unless a very slow

baud rate is used, the printer will get	out of synchronization.

To get a hardware handshake, add a jumper from pin 3 to	pin 22 of the

8251 UART and test for ready on	bit D7 (DSR) of	the status word.

8255 Parallel Interface	--

H&H Board, Z-89-11 Board, Environmental	Control	PC-12/89

To use the parallel interface in the H&H board, Z89-11 parallel port,

or Environmental Control PC-12/89, use MX8255.DVD renamed to LP.DVD.

Driver Reassembly (OPTIONAL):

If you want to reassemble the driver, change the Z8255 assembly	switch

to FALSE and all other interface switches to TRUE. (File LPSWITCH.ACM)

The printer is connected to the	computer as follows:

Printer	8255	       H&H     PC-89

Signal	Signal Printer "D"     PC-12   HCA-14

-------	------ ------- ------- ------- ------

STROBEA	PC-0 . .1. . . .9. . . PA-10* .	1

Data0. .PA-0 . .2. . . .1. . . PA-4. . .3

Data1. .PA-1 . .3. . . .2. . . PA-11 . .5

Data2. .PA-2 . .4. . . .3. . . PA-3. . .7

Data3. .PA-3 . .5. . . .4. . . PA-12 . .9

Data4. .PA-4 . .6. . . 14. . . PA-2. . 11

Data5. .PA-5 . .7. . . 15. . . PA-13 . 13

Data6. .PA-6 . .8. . . 16. . . PA-1. . 15

Data7. .PA-7 . .9. . . 17. . . PA-14 . 17

ACKNLG .PC-6 . 10. . . 24. . . PA-5. . 19

BUSY . .PC-4 . 11. . . 22. . . PA-6. . 21

Ground . . . . 33. . . 13

Ground . . . . 25. . . . . . . PA-8

Ground . . . . 16. . . . . . . . . . . 23

Ground . . . . 16. . . . . . . . . . . 24

* On PC-12/89 connect PA-10 to PB-14 to	use one	connector

and make sure	pin 14 of the printer is NOT grounded.

Initial	8255 related SET options:

PORT	320 for	TD, Zenith, & H&H; 004 for PC-12/89

MASK	020 (Test port C, bit 4	for printer ready/busy)

READY	000 (Low for ready, High for busy)

The 8255 control word is 250 (Port A, mode 1 out; Port B, not used;

and Port C, mode 0 with	bits 4 & 5 as inputs)

The bit	mask is	used to	select the bits	to test	for printer busy.

When the signal	from the printer matches the comparison	word, the

printer	is ready to receive another input from the printer driver.

As set,	bits 4 and 5 of	port C are inputs with bit 4 used for the

busy handshake and bit 5 unused.

8255						 PRINTER

SIGNAL						 SIGNAL

______________________________

DATA0/7	____|______________________________  --> DATA0/7

___________	    _______________

PC-0		   |_______|		     --> STROBE*

_________

PC-4	_____________|	       |___________ <--	 BUSY

__________________    _____________

PC-6 ACK*			  |__|		    <--	 ACK*

(STROBE* and ACK* indicate Low-True levels on STROBE and ACK lines)

Data is	placed on the data lines.

The STROBE line	is pulsed low.

BUSY from printer goes high.

When printer is	done, it issues	ACK* which resets OBF*.

BUSY from printer goes low.

8255 Parallel Interface	--

Tru-Data Parallel Interface Board

Reassemble the driver with the U8255 assembly switch set to TRUE, and

all other interface switches FALSE.

The printer is connected to the computer as follows:

Ribbon	Tru-Data

Signal	Printer	Cable	P-603

ACKNLG	10	1	26	(PC-6)

STROBEA	1	2	28	(PC-7)

Data0	2	3	30	(PA-0)

Data1	3	4	32	(PA-1)

Data2	4	5	34	(PA-2)

Data3	5	6	36	(PA-3)

Data4	6	7	38	(PA-4)

Data5	7	8	40	(PA-5)

Data6	8	9	42	(PA-6)

Data7	9	10	44	(PA-7)

BUSY	11	11	22	(PC-4)*

* Install jumper from U8 pin 4 to Pad "X" on board.

With this arraingement,	Port "B" is available for any other use.

MASK	Port C bit mask		    020	(Test bit 4)

READY	Port C comparison word	    020	(Low here for busy)

8255 Control Word		    250	(Port A, mode 1	out

Port B, not used

Port C, mode 0	in)

The bit	mask is	used to	select the bits	to test	for printer busy.

When the signal	from the printer matches the comparison	word, the

printer	is ready to receive another input from the printer driver.

As set,	Bits 4 and 5 of	port C are inputs. If you want to add paper

sensing, wire pad "Y" to pin 11	of U2 (PC-5) and change	the port C

mask and comparison words.

"Direct" Parallel Interface --

FBE Parallel Board or Do-It-Yourself Parallel Interface

Driver Reassembly.

Change assembly	switch DIRECT to TRUE and all other interface switches

to FALSE (in LPSWITCH.ACM)

PERCOM ZFD Dual	Density	Disk Controller	Printer	Port

Driver Reassembly.

Change assembly	switch PERCOM to TRUE and all other interface switches

to FALSE (in LPSWITCH.ACM)

Hardware Installation.

On your computer's Serial Interface Card, remove the 8250 UART IC

from location U602. (This is a large chip near the board's top center.)

This eliminates contention for port 340.

Make the following connections: (G=Ground)

PERCOM	Printer

34 pin	36 pin	Signal		USE (port)

------	------	-------------	----------

1	 1	STROBE		HARDWARE GENERATED OUTPUT STROBE

2	19	G

3	 2	DATA 0		OUT BIT 0 (340)

4	20	G

5	 3	DATA 1		OUT BIT 1 (340)

6	21	G

7	 4	DATA 2		OUT BIT 2 (340)

8	22	G

9	 5	DATA 3		OUT BIT 3 (340)

10	23	G

11	 6	DATA 4		OUT BIT 4 (340)

12	24	G

13	 7	DATA 5		OUT BIT 5 (340)

14	25	G

15	 8	DATA 6		OUT BIT 6 (340)

16	26	G

17	 9	DATA 7		OUT BIT 7 (340)

18	27	G

19	10	ACK		IN BIT 2 (345) NOT USED

20	28	G

21	11	BUSY		IN BIT 5 (345) HIGH = BUSY

22	29	G

23	12	PAPER OUT	IN BIT 4 (345) HIGH = NO PAPER

24	30	G

25	13	SELECT		IN BIT 0 (345) HIGH = SELECTED

26	31	INIT		NOT USED

27	14	AUTO LINE FEED	* SEE BELOW

28	32	FAULT		IN BIT 1 (345) LOW = PRINTER FAULT

29	15	NOT USED

30	33	G

31	16	LOGIC GROUND

32	34	NOT USED

33	17	CHASSIS GROUND

34	35	NOT USED

NONE	18	NOT USED

NONE	36	NOT USED

* If you want to do overstrikes	or underlining,	this line must be broken.

Either cut the etch on the top of the board or leave it	unconnected in

the printer cable.