LINDLEY SYSTEMS			   V3.4

21 Hancock St.		       03/04/85

Bedford, Ma. 01730

(617) 275-6821

USER PROGRAMMABLE CHARACTER BIOS MODIFICATION

USER'S GUIDE

Special printer features can be programmed by use of the character

conversion routines built into the driver.  The backslash character is

reserved as a control code conversion character. When this character is

encountered, it sets an internal switch in the driver and the current

character is deleted. The next character is then examined; if it is

another conversion character, it is sent unchanged to the printer.  If

it is the @ character, it is deleted and an internal switch set to

delete the next new line character.  If it is any other character, an

escape code is produced.

The following are the codes recognized by the driver:

\C	    Cursor forward (produces one space)

\F	    Starts the graphics mode

\G	    Ends the graphics mode

\H	    Home - starts a new picture

\L	    produces a form feed

\N	    Normal width characters	(initial size)

\p	    Starts the inverse print mode (note lower case p)

\q	    Ends the inverse print mode   (note lower case q)

\S	    Single height characters	(initial size)

\T	    Double height characters

\Yrc    Cursor addressing (works only down and forward)

r = row character, c = column character

\Wn     Width control  n = 1, 2, 3, or 4

\1	    Dot density 1

\2	    Dot density 2

\3	    Dot density 3

\4	    Dot density 4

\6	    Six colunms per character to produce 80 characters/line,

\8 is the default mode

\8	    Eight colunms per character to produce 106 characters/line

\6 must be programmed in the data sent to the printer

\@	    Deletes the next new line character (extends line)

\\	    sends itself to the printer

Note: \1 to \8, \N, \S \W, and \T act on the current text line.

That is they hold for the entire line.  If there is a conflict

on any line, the last command of the type in conflict is the

one that holds for the line.	If the conflict occurs after the

maximum number of characters per line for the selected mode has

been sent to the printer, the line is cut off at the maximum

number of characters and the remaining characters will be lost.

In addition to the above codes, ESC F, ESC G, ESC p, and ESC q produce

the same printed results as seen on the screen. Be sure to notice that

only the escape codes in the above are recognized by the driver.  All

other escape codes are deleted.

Horizontal tab characters are deleted by the driver and spaces inserted

to match the HEATH standard of every eight columns.

The program TP can be used for line by line typing to the LST: device.

If LST: is set to UL1, graphic characters will be printed, if LST: is

set to LPT:, normal printing is done. It allows the use of all the CP/M

command line features for editing the line.  Nothing is printed until

you type return.  At this time the entire current line is printed.  CP/M

changes the form of all control codes, but the driver will make the

correct conversion to produce the correct graphics and characters.

This driver is meant to augment the normal printer driver. The use of

the dot graphic print mode requires two unidirectional passes of the

print head to print each line. Thus, the print speed is reduced by

about a factor of four.  However, it opens up a whole new world of

interesting possibilities that require special effects.  You are not

limited to one character set! You can create any number of special

drivers, each with its own character set, to be used as the need arises.

You may, for example, have several copies of the PRGR program (each with

a unique name), each containing a separate character set.

You will find instructions for using the character generator in

file CHR8X12.DOC.

The conversion character may be changed to any value by modifying the

source code. Just look up the octal value of the desired character and

enter that value. The value 377 will disable conversion. (Use ASCII to

octal chart.)

This driver is installed as UL1: in your BIOS.	It, in turn, uses the

LST: code in the BIOS to send each character to the printer.  To

activate the UPC matrix graphics, either reference UL1: directly, or

use STAT LST:=UL1: to redirect the listing data to it.

The ED-A-SKETCH program (available from the Software Toolworks) may be

used to edit 80 column wide pictures. Be sure to save the sketch in the

picture format.  Be sure that the print lines that you make up are not

longer than the selected print style maximum number of characters per line.

#

colunms per character to produc