04/23/84

For the normal Heath/Zenith serial interface, copy U8250.DVD to your

system disk and rename it LP.DVD.

Serial printer connections:

Use either a one to one wired standard RS232 cable, a cable with

wires on pins 1 through 8 and pin 20, or simply a three wire

cable.

Printer   H89	H8-4	Signal

3	   3	 5	Received data

7	   7	13	Signal ground

20	  20	14	Data Set Ready

Alternate connection with crossed busy/ready line and reversed

ready signal.

Printer   H89	H8-4	Signal

3	   3	 5	Received data

7	   7	13	Signal ground

20	   4	 7	Clear To Send

This connection requires the use of the set command to set the

MASK to 020 and READY to 000.

Data Set Ready mask is bit pattern 040 which monitors RS232 pin 20.

Ready bits 040 means a high signal for ready, 000 for low ready.

Clear To Send mask is bit pattern 020 which monitors RS232 pin 4.

Ready bits 020 means a high signal for ready, 000 for low ready.