~[L003~@

04/23/83

To use the 8255 parallel interface in the H&H board, Z89-11 parallel

port, or Environmental Control PC-12/89 copy Z8255.DVD to your system

disk and rename it LP.DVD.

To reassemble the driver, set the Z8255 assembly switch ON and the

other interface switches to OFF.  Use these connections:

Printer 8255	       H&H     PC-89

Signal	Signal Printer "D"     PC-12   HCA-14

------- ------ ------- ------- ------- ------

STROBEA PC-0 . .1. . . .9. . . PA-10* . 1

Data0. .PA-0 . .2. . . .1. . . PA-4. . .3

Data1. .PA-1 . .3. . . .2. . . PA-11 . .5

Data2. .PA-2 . .4. . . .3. . . PA-3. . .7

Data3. .PA-3 . .5. . . .4. . . PA-12 . .9

Data4. .PA-4 . .6. . . 14. . . PA-2. . 11

Data5. .PA-5 . .7. . . 15. . . PA-13 . 13

Data6. .PA-6 . .8. . . 16. . . PA-1. . 15

Data7. .PA-7 . .9. . . 17. . . PA-14 . 17

ACKNLG .PC-6 . 10. . . 24. . . PA-5. . 19

BUSY . .PC-4 . 11. . . 22. . . PA-6. . 21

Ground . . . . 33. . . 13

Ground . . . . 25. . . . . . . PA-8

Ground . . . . 16. . . . . . . . . . . 23

Ground . . . . 16. . . . . . . . . . . 24

* On PC-12/89 connect PA-10 to PB-14 to use one connector

and make sure pin 14 of the printer is NOT grounded.

Initial 8255 related SET options:

PORT    320 for TD, Zenith, & H&H; 004 for PC-12/89

MASK    020 (Test port C, bit 4 for printer ready/busy)

READY   000 (Low for ready, High for busy)

The 8255 control word is 250 (Port A, mode 1 out; Port B, not used;

and Port C, mode 0 with bits 4 & 5 as inputs)

The bit mask is used to select the bits to test for printer busy.

When the signal from the printer matches the comparison word, the

printer is ready to receive another input from the printer driver.

As set, bits 4 and 5 of port C are inputs with bit 4 used for the

busy handshake and bit 5 unused.

8255						 PRINTER

SIGNAL						 SIGNAL

______________________________

DATA0/7	____|______________________________  --> DATA0/7

___________	    _______________

PC-0		   |_______|		     --> STROBE*

_________

PC-4	_____________|	       |___________ <--  BUSY

__________________    _____________

PC-6 ACK*			  |__|		    <--  ACK*

Data is placed on the data lines.

The STROBE line is pulsed low.

BUSY from printer goes high.

When printer is done, it issues ACK* which resets OBF*.

BUSY from printer goes low.~[L000~L