~[L003~@

04/23/84

To use the Tru-Data Parallel interface board, copy U8255.DVD to your

system disk and rename it to LP.DVD.

If you want to reassemble the driver, set the U8255 assembly switch to

ON and the other interface switches set to OFF.

The printer is connected to the computer as follows:

Ribbon	Tru-Data

Signal	Printer Cable	P-603

ACKNLG	10	1	26	(Port C, pin 6)

STROBEA 1	2	28	(Port C, pin 7)

Data0	2	3	30	(Port A, pin 0)

Data1	3	4	32	(Port A, pin 1)

Data2	4	5	34	(Port A, pin 2)

Data3	5	6	36	(Port A, pin 3)

Data4	6	7	38	(Port A, pin 4)

Data5	7	8	40	(Port A, pin 5)

Data6	8	9	42	(Port A, pin 6)

Data7	9	10	44	(Port A, pin 7)

BUSY	11	11	22	(Port C, pin 4)*

Grounded pins not listed

* Install jumper from U8 pin 4 to Pad "X" on board.

Initial Tru Data related SET options:

MASK    Port C bit mask		    020 (Test bit 4)

READY   Port C comparison word	    000 (Low here for busy)

8255 Control Word		    250 (Port A, mode 1 out

Port B, not used

Port C, mode 0 in)

The bit mask is used to select the bits to test for printer busy.

When the signal from the printer matches the comparison word, the

printer is ready to receive another input from the printer driver.

As set, Bits 4 and 5 of port C are inputs.	If you want to add paper

sensing, wire pad "Y" to pin 11 of U2 (PC-5) and change the port C

mask and comparison words.~[L000~L