Interface Guide

for

Lindley Systems

Printer Drivers

For the usual Heath/Zenith serial interface:

Use either a one to one wired standard RS232 cable, a cable with

wires on pins 1 through 8 and pin 20, or simply a three wire

cable as shown below (Set MASK to 040 and READY to 040):

Printer   H89                           H8-4 Molex connector

3      3   Received Data              5

7      7   Signal Ground             13

20     20   Data Set Ready            14

Alternate connection with crossed busy/ready line and reversed ready

signal is shown below (Set MASK to 020 and READY to 000):

Printer   H89                           H8-4 Molex connector

3      3   Received Data              5

7      7   Signal Ground             13

20      4   Data Set Ready             7

Note: This alternate wiring is compatible with some Heath/Zenith

drivers, such as LPH24.DVD and CP/M BIOS, but needs a special cable.

You can also jumper pins 4 and 11 of the RS232 connecter in the

printer and wire pins 3, 4, 7, and 20 between the computer and

printer to be compatible with Magnolia CP/M.

H-8-2 Parallel Port:

PRINTER TO H8-2 CABLE CONNECTIONS

PARALLEL             P-101

PRINTER       PIN             PIN          H8-2

-----------------             -----------------

/STROBE. . . .  1 ----------- 11  TAKE DATA

* 19 -----------  9  *

DATA 1 . . . .  2 -----------  1  DATA 1

* 20 -----------  9  *

DATA 2 . . . .  3 -----------  2  DATA 2

* 21 -----------  9  *

DATA 3 . . . .  4 -----------  3  DATA 3

* 22 -----------  9  *

DATA 4 . . . .  5 -----------  4  DATA 4

* 23 -----------  9  *

DATA 5 . . . .  6 -----------  5  DATA 5

* 24 -----------  9  *

DATA 6 . . . .  7 -----------  6  DATA 6

* 25 -----------  9  *

DATA 7 . . . .  8 -----------  7  DATA 7

* 26 -----------  9  *

DATA 8 . . . .  9 -----------  8  DATA 8

* 27 -----------  9  *

BUSY . . . . . 11 ----------- 10  DATA TAKEN

* 29 -----------  9  *

* Indicates ground wires if ribbon cable is used.

Otherwise, use appripriate signal return.

Data lines are high true.

/STROBE is true low.  When this signal goes from high to low, the

printer accepts the data present on the data lines.

DATA TAKEN completes the parallel output action and sets the TXRDY

bit to show that the 8251 can take another byte.

The 8251 Transmitter ready bit is used to test for ready.

For the H8-2 parallel interface, set the the jumpers as follows:

A1 - A2 OPEN            All interrupts open.

B1 - B2 CONNECTED

C1 - C2 CONNECTED

C1 - C3 OPEN

E1 - E2 CONNECTED

E1 - E3 OPEN

F1 - F2 OPEN

G1 - G2 OPEN

H1 - H2 OPEN

H-8-5 Serial Port:

With the H8-5 board, no handshake is provided.  Unless a very

slow baud rate is used, the printer will get out of

synchronization. To get a hardware handshake, add a jumper from

pin 3 to pin 22 of the 8251 UART and test for ready on bit D7

(DSR) of the status word.

H&H Parallel, Environmental Control PC-12/89, and Z-89-11 Boards:

Use the version of the driver with the Z8255 interface.

To reassemble the driver, change the Z8255 assembly switch to

YES and the other interface switches to NO.

The printer is connected to the computer as follows:

Printer 8255           H&H     PC-89

Signal  Signal Printer "D"     PC-12   HCA-14

------- ------ ------- ------- ------- ------

STROBEA PC-0 . .1. . . .9. . . PA-10* . 1

Data0. .PA-0 . .2. . . .1. . . PA-4. . .3

Data1. .PA-1 . .3. . . .2. . . PA-11 . .5

Data2. .PA-2 . .4. . . .3. . . PA-3. . .7

Data3. .PA-3 . .5. . . .4. . . PA-12 . .9

Data4. .PA-4 . .6. . . 14. . . PA-2. . 11

Data5. .PA-5 . .7. . . 15. . . PA-13 . 13

Data6. .PA-6 . .8. . . 16. . . PA-1. . 15

Data7. .PA-7 . .9. . . 17. . . PA-14 . 17

ACKNLG .PC-6 . 10. . . 24. . . PA-5. . 19

BUSY . .PC-4 . 11. . . 22. . . PA-6. . 21

Ground . . . . 33. . . 13

Ground . . . . 25. . . . . . . PA-8

Ground . . . . 16. . . . . . . . . . . 23

Ground . . . . 16. . . . . . . . . . . 24

* On PC-12/89 connect PA-10 to PB-14 to use one connector

and make sure pin 14 of the printer is NOT grounded.

Initial 8255 related SET options:

PORT    320 for TD, Zenith, & H&H; 004 for PC-12/89

MASK    020 (Test port C, bit 4 for printer ready/busy)

READY   000 (Low for ready, High for busy)

The 8255 control word is 250 (Port A, mode 1 out; Port B, not used;

and Port C, mode 0 with bits 4 & 5 as inputs)

The bit mask is used to select the bits to test for printer busy.

When the signal from the printer matches the comparison word, the

printer is ready to receive another input from the printer driver.

As set, bits 4 and 5 of port C are inputs with bit 4 used for the

busy handshake and bit 5 unused.

This chart indicates the flow of signals. 'Time' reads from left

to right.  First, data is placed on the data lines.  Then,

the STROBE signal is pulsed low. BUSY from the printer goes high.

When the printer is done, it asserts ACK* (low true) which resets

OBF*.  Finally, BUSY from the printer goes low, indicating it is

ready for another character.

8255                                                 PRINTER

SIGNAL                                               SIGNAL

______________________________

DATA0/7	____|______________________________  --> DATA0/7

___________         _______________

PC-0		   |_______|                 --> STROBE*

_________

PC-4	_____________|         |___________ <--  BUSY

__________________    _____________

PC-6 ACK*		  |__|              <--  ACK*

Tru-Data Parallel interface board:

Reassemble the driver with the U8255 assembly switch set to YES

and the other interface switches set to NO.

The printer is connected to the computer as follows:

MX      Ribbon  Tru-Data

Signal  Printer Cable   P-603

ACKNLG  10      1       26      (PC-6)

STROBEA 1       2       28      (PC-7)

Data0   2       3       30      (PA-0)

Data1   3       4       32      (PA-1)

Data2   4       5       34      (PA-2)

Data3   5       6       36      (PA-3)

Data4   6       7       38      (PA-4)

Data5   7       8       40      (PA-5)

Data6   8       9       42      (PA-6)

Data7   9       10      44      (PA-7)

BUSY    11      11      22      (PC-4)*

* Install jumper from U8 pin 4 to Pad "X" on board.

With this arraingement, Port "B" is available for any other use.

MASK    Port C bit mask                 020 (Test bit 4)

READY   Port C comparison word          020 (Low here for busy)

8255 Control Word               250 (Port A, mode 1 out

Port B, not used

Port C, mode 0 in)

The bit mask is used to select the bits to test for printer busy.

When the signal from the printer matches the comparison word, the

printer is ready to receive another input from the printer driver.

As set, Bits 4 and 5 of port C are inputs.  If you want to add paper

sensing, wire pad "Y" to pin 11 of U2 (PC-5) and change the port C

mask and comparison words.

FBE Parallel Board:

Follow FBE installation instructions.  Re-assemble the driver

with the DIRECT switch set to YES and all other interface

switches set to NO.

Percom ZFD Disk Controller Board Printer Port:

Remove the 8250 IC from U602 (large IC at top center of board).

This eliminates contention for port 340 which is used for parallel

port operations from this board.

Make the following connections:

PERCOM  Printer

34 pin  26 pin  Signal          USE (port)

------  ------  ------          ----------

1       1       STROBE          HARDWARE GENERATED OUTPUT STROBE

2       19      G

3       2       DATA 0          OUT BIT 0 (340)

4       20      G

5       3       DATA 1          OUT BIT 1 (340)

6       21      G

7       4       DATA 2          OUT BIT 2 (340)

8       22      G

9       5       DATA 3          OUT BIT 3 (340)

10      23      G

11      6       DATA 4          OUT BIT 4 (340)

12      24      G

13      7       DATA 5          OUT BIT 5 (340)

14      25      G

15      8       DATA 6          OUT BIT 6 (340)

16      26      G

17      9       DATA 7          OUT BIT 7 (340)

18      27      G

19      10      ACK             IN BIT 2 (345) NOT USED

20      28      G

21      11      BUSY            IN BIT 5 (345) HIGH = BUSY

22      29      G

23      12      PAPER OUT       IN BIT 4 (345) HIGH = NO PAPER

24      30      G

25      13      SELECT          IN BIT 0 (345) HIGH = SELECTED

26      31      INIT            NOT USED

27      14      AUTO LINE FEED  * SEE BELOW

28      32      FAULT           IN BIT 1 (345) LOW = PRINTER FAULT

29      15      NOT USED

30      33      G

31      16      LOGIC GROUND

32      34      NOT USED

33      17      CHASSIS GROUND

34      35      NOT USED

NONE    18      NOT USED

NONE    36      NOT USED

G INDICATES GROUND CONNECTION ABOVE

* If you want to do overstrikes or underlining, this line must be broken.

Either cut the etch on the top of the board or leave it unconnected in

the printer cable.

Do-It-Yourself Parallel Port:

Follow construction instructions below.  Re-assemble the driver

with the DIRECT switch set to YES and all other interface

switches set to NO.

----------------------------------------------------------------------

DO-IT-YOURSELF Parallel Printer Output Port

Printer Parallel input plug            Components on perforated board

via ribbon cable

19 GND                                  74LS244                H-89

P 504

1 STROBE ---->STROBE         +5<----- 20  ,-- 1 --------GND----- 1

|

20 GND                                 19--'   2 --------DATA 0-- 2

2 DATA 0 -----------------------------18 ,----3   ,-----DATA 1-- 3

,-----'  ,-----'

3 DATA 1 --------------------------'  17----' 4 --------DATA 2-- 4

4 DATA 2 -----------------------------16 ,----5   ,-----DATA 3-- 5

,-----'  ,-----'

5 DATA 3 --------------------------'  15----' 6 --------DATA 4-- 6

6 DATA 4 -----------------------------14 ,----7   ,-----DATA 5-- 7

,-----'  ,-----'

7 DATA 5 --------------------------'  13----' 8 --------DATA 6-- 8

8 DATA 6 -----------------------------12 ,----9   ,---,-DATA 7-- 9

,-----'  ,-----'   |

9 DATA 7 --------------------------'  11----'10 --GND | GND---- 10

|

28 GND       ,--------------------------------------,  |

|                          74LS125     |  |

11 BUSY -----'                  +5<--- 14   ,--1    |  |

13   |  2 ---'  |

29 GND                                 12   |  3 ------'

11   |  4

10   |  5

9   |  6

8   |  7 -- GND

|

|

7432      |                 P-510

+5<--- 14   |  1 -----BRD------ 6

13   |  2 --,

12   '--3   |--PORT----- ****

**** PORT   PIN                      11      4 --'

340     11                      10      5 -----BWR------ 7

330     10                       9   ,--6

320      9                       8   |  7 -----GND-,---- 2

|             |

|     .1 UF --'--

STROBE <-----------'     CAP.  --,--

|

+5 <-----'---- 1

P-505 AND P-511 MAY BE USED INSTEAD OF P-504 AND P509

Note: No clearance on back of board for wire wrap connections, use

point to point wiring.

PARTS LIST

1       APPROXIMATELY 4 1/2 X 4 1/2 IN. PERFORATED BOARD WITH .1" GRID HOLES

1       7432 OR GATE

1       74LS125 TRI-STATE BUS GATE

1       74LS244 OCTAL BUS DRIVER

6 FT.   14 WIRE RIBBON CABLE

1       .1 UF  50 VOLT CAPACITOR

1       20 PIN SOLDER TAIL DIP SOCKET

2       14 PIN SOLDER TAIL DIP SOCKET

1       36 PIN CONNECTOR WITH COVER, AMPHENOL 57-30360

1       10 PIN BOARD CONNECTOR, HEATHKIT 432-1074

1       25 PIN BOARD CONNECTOR, HEATHKIT 432-1076

The 10 and 25 pin board connectors are .25" apart.  This means that you

will have to drill 10 .04" holes between the existing .1" spaced holes to

mount the 10 pin connector.   Look at the computer board to determine the

connector positions.

Put a drop of glue under each IC socket to hold it in place while you work.

Port 320 is least likely to cause problems.  This corresponds to the unused

port position on some H88-3 serial boards.  In case this port has the 8250

inserted, it must be removed to prevent both ports being on the bus at one

time.

IMPORTANT NOTICE: This is a minimum configration.  It may cause some

television interference.   Radiation can be reduced by using a 26 wire

ribbon cable with every other wire grounded.  Additional attenuation

can be achieved by adding additional gates so that only I/O to the

printer port is sent down the line.  Here, all bus activity is going

down the wire.  To quiet it even more, connect a 47 ohm resistor in

series with each lead going to the printer.  If you can find ferrite

beads about 1/4" long, run each output wire through one as it leaves

the board.  These work even better than the 47 ohm resistors for

television interference reduction.