~[L003~@

04/23/84

For use with the H8-2 parallel or H8-5 serial board with 8251 UART:

Copy driver U8251.DVD to your system disk and rename it LP.DVD.

PRINTER TO H8-2 CABLE CONNECTIONS

PARALLEL	      P-101

PRINTER       PIN	      PIN	   H8-2

-----------------	      -----------------

/STROBE. . . .	1 ----------- 11  TAKE DATA

* 19 -----------  9  *

DATA 1 . . . .	2 -----------  1  DATA 1

* 20 -----------  9  *

DATA 2 . . . .	3 -----------  2  DATA 2

* 21 -----------  9  *

DATA 3 . . . .	4 -----------  3  DATA 3

* 22 -----------  9  *

DATA 4 . . . .	5 -----------  4  DATA 4

* 23 -----------  9  *

DATA 5 . . . .	6 -----------  5  DATA 5

* 24 -----------  9  *

DATA 6 . . . .	7 -----------  6  DATA 6

* 25 -----------  9  *

DATA 7 . . . .	8 -----------  7  DATA 7

* 26 -----------  9  *

DATA 8 . . . .	9 -----------  8  DATA 8

* 27 -----------  9  *

BUSY . . . . . 11 ----------- 10  DATA TAKEN

* 29 -----------  9  *

* Indicates ground wires if ribbon cable is used.

Otherwise, use appripriate signal return.

Data lines are high true.

/STROBE is true low.  When this signal goes from high to low, the

printer accepts the data present on the data lines.

DATA TAKEN completes the parallel output action and sets the TXRDY

bit to show that the 8251 can take another byte.

The 8251 Transmitter ready bit is used to test for ready.

For the H8-2 parallel interface, set the the jumpers as follows:

A1 - A2 OPEN		All interrupts open.

B1 - B2 CONNECTED

C1 - C2 CONNECTED

C1 - C3 OPEN

E1 - E2 CONNECTED

E1 - E3 OPEN

F1 - F2 OPEN

G1 - G2 OPEN

H1 - H2 OPEN

With the H8-5 board, no handshake is provided.	Unless a very slow

baud rate is used, the printer will get out of synchronization.

To get a hardware handshake, add a junper from pin 3 to pin 22 of the

8251 UART and test for ready on bit D7 (DSR) of the status word.~[L000~L