05/11/85

To use the 8255 parallel interface on the H89PIP board, set the H89PIP

assembly  switch at the beginning of the source file GRV35.ASM to YES

and all the others in that group to NO.  Follow the assembly instructions

in the main documentation file.

The printer is connected to the computer as given in the FBE instructions.

Initial 8255 related SET options:

LPPORT    320

LPMASK    004

LPCOMP    000 (Low for ready, High for busy)

These may be changed in the assembly code and the driver reassembled.