~[L003~@

04/23/84

For interface to the PERCOM ZFD dual density disk controller printer port:

You must assemble the driver by setting the PERCOM assembly switch to ON

and all the other interface switches to OFF.

Remove the 8250 IC from U602 (large IC at top center of board).

This eliminates contention for port 340 which is used for parallel

port operations from this board.

Make the following connections:

PERCOM	PRINTER

34 pin	36 pin	Signal		USE (port)

------	------	------		----------

1	1	STROBE		HARDWARE GENERATED OUTPUT STROBE

2	19	G

3	2	DATA 0		OUT BIT 0 (340)

4	20	G

5	3	DATA 1		OUT BIT 1 (340)

6	21	G

7	4	DATA 2		OUT BIT 2 (340)

8	22	G

9	5	DATA 3		OUT BIT 3 (340)

10	23	G

11	6	DATA 4		OUT BIT 4 (340)

12	24	G

13	7	DATA 5		OUT BIT 5 (340)

14	25	G

15	8	DATA 6		OUT BIT 6 (340)

16	26	G

17	9	DATA 7		OUT BIT 7 (340)

18	27	G

19	10	ACK		IN BIT 2 (345) NOT USED

20	28	G

21	11	BUSY		IN BIT 5 (345) HIGH = BUSY

22	29	G

23	12	PAPER OUT	IN BIT 4 (345) HIGH = NO PAPER

24	30	G

25	13	SELECT		IN BIT 0 (345) HIGH = SELECTED

26	31	INIT		NOT USED

27	14	AUTO LINE FEED	* SEE BELOW

28	32	FAULT		IN BIT 1 (345) LOW = PRINTER FAULT

29	15	NOT USED

30	33	G

31	16	LOGIC GROUND

32	34	NOT USED

33	17	CHASSIS GROUND

34	35	NOT USED

NONE	18	NOT USED

NONE	36	NOT USED

G INDICATES GROUND CONNECTION ABOVE

* If you want to do overstrikes or underlining, this line must be broken.

Either cut the etch on the top of the board or leave it unconnected in

the printer cable.~L000~L