Page 4

RUNPC - Runs Certain IBM PC Programs

(C) 1983 Lindley Systems

21 Hancock St.

Bedford, MA 01730

(617) 275-6821

RUNPC.COM

This program simulates some of the IBM PC interrupt instructions.

The  functions	that  have  been  implemented are the portions of

interrupt services 10, 11, 12, 16, and 17 that are needed to  run

programs  written  in  Ashton-Tate's  DBase  II (tm) language and

other IBM PC programs, such as	the  Perfect  series,  and  Micro

Pro's Wordstar (tm).

To install RUNPC, use the COPY command to copy the RUNPC.COM file

to  your  system  diskette,  as follows: (Put your system disk in

drive A:, if it is not already there, and the RUNPC disk in drive

B:)

COPY B:RUNPC.COM A:

Whenever you wish to set your computer up as an IBM, simply enter

the  command "RUNPC" from the Z-DOS command mode (the A: prompt),

as follows:

A:RUNPC

The RUNPC program will clear the  screen  and  and  announce  its

presence.  From  this  point  on,  you	may  simply  run your IBM

programs as if you were running on an IBM machine.

Note that some IBM programs will not run due  to  the  particular

hardware constraints of the Zenith computer.  The following types

of programs are the basic categories which will not run:

* Modem or telecommunications programs.

These will not run,  since  the  asynchronous  communi-

cations element on the Zenith is different from that in

the IBM.

* Graphics programs.

These  will  not  run,  as  the  Zenith  graphics   are

different from those on the IBM.  The price you pay for

a flicker-free, full eight color, 640 x 225 display  is

incompatibility with the IBM.

* Programs which heavily rely on the IBM keyboard.

Again, the Zenith has a different processor to  control

the keyboard.

Some programs which require interrupts or services  not  possible

with  the  Heath/Zenith hardware or services not provided by this

program will operate strangely.  Be  sure  to  test  the  program

operation before entering valuable data.  Lindley Systems assumes

no liability for the operation of this program.  If you have  any

problems, please contact us and we will try to find a solution.